CSE321 Lab environment

- Using Nickelback in CSE department

  - Log on to the nickelback.cse.buffalo.edu

  - cd /projects/bina/NEXOS/CSE321/

  - Make your own directory with your UBIT name. You have to do it only once.

  - Copy a set of xinu source file(xinu_mips-latest.gz) to your own directory in NEXOS. “gunzip < file.tar.gz | tar xvf –“ command should work with the gz file.

  - You should modify the compile/makeVars file. The “MIPS_ROOT” variable should be changed

from “/usr/local/project/mipsel-dev/bin”

to “/projects/bina/NEXOS/CSE321/project/mipsel-dev/bin”

  - Modify, compile and do whatever you want with your source codes.

- Using lab PCs in 206 Fronzak

  - Log on to one of the three Linux machines with your UBIT name and person #.

  - Make your own xinu directory in your space.

  - Modify and compile the xinu.boot image(You don’t need to change the makeVars file but check it out at least once).

  - Transfer the boot image from the Linux machine to the Windows machine using any kind of media(USB memory stick or floppy diskette)

  - Make sure you’ve done all things listed below before downloading the xinu.boot onto your WRT54GL.

    - Connect your WRT54GL router to the main router. Check out all physical connection among devices.

    - Check out IP address of the Windows machine given by the main router(You can use ipconfig or etc. to check IP). It should be something like 192.168.0.xxx.

    - Make sure that you execute the TFTP server software before the downloading. Sometimes the TFTP server stops working due to a timeout or etc. So see whether it’s working well after a long break.

    - Start the serial communication and do needed things discussed in class. Don’t forget to execute “ifconfig” command before “boot” command.

- Miscellaneous

  - Refer to the website http://xinu.mscs.mu.edu/ and Readme file in the xinu source file directory whenever you need any help.

  - Don’t change anything outside your own working directory.

  - Log off when you don’t use a machine in the lab for a long time for others. No one is responsible for your unsaved files.