NEXOS LAB GUIDELINE

- Log in on any computer in the lab

- Start Putty

- Type the host-name as ‘nexos-dlink.cse.buffalo.edu’

- Enter your ubit-name and password (your password is your person #)

- Go to the folder where you have the .boot file

- Now the cross compiler installed in the NEXOS server is not available. You can cross compile your xinu in nickelback. To transfer a xinu.boot image from nickelback to nexos-dlink, follow below steps.

  - Start a new Putty and login to nickelback.cse.buffalo.edu

  - Go to the folder where your boot image is.

  - Type this command in the prompt.

    - sftp nexos-dlink.cse.buffalo.edu

  - Enter your password for nexos-dlink server(your person#)

  - The initial current directory in the nexos-dlink is

/home/your_ub_it_name.

  - You can change the directory using cd.

  - Type this command to transfer the boot image to nexos-dlink server.

    - put filename.boot

    - If you didn’t change the destination directory, your file is in your home directory in the nexos-dlink server.

- Transfer the file to the folder ‘/var/lib/tftpboot/ by typing the command below

*/>  cp  filename.boot  /var/lib/tftpboot/

- If you have permission denied error, try below command.

  - chgrp cse321 filename.boot

- The file should be cse321 group accessible.

  - Check it using “ls -l” command and see whether or not the group id(after your ubit name in the result) is cse321.

- You should name your boot image with your ubit name so that your boot image has unique name in the directory. For example, mine could be xinu_dyou.boot. Otherwise, you may not be able to copy your boot image to the target directory.

- Type the command “xinu-console  <target-name>”

  - Current target names are stump, waggonner, moiz, hartnett, and Russell.

  - The target-names are the names of the routers you are connecting to.

  - The target-name and the IP-address of the router is on the side of each of the routers.

- This step would require more than one person to perform if you are not close to the server.

  - Suppose you typed stump as the target-name, one person (probably your partner) would hold the keys ‘Ctrl+c’, and another person will go to the router named stump, and disconnect and reconnect the power supply.

  - This process will start the CFE prompt.

- Type below two commands to boot xinu. These are basically the same commands as we typed in basic unit.

  - CFE> ifconfig eth0 –auto

  - CFE> boot –elf 192.168.0.100:filename.boot

- 192.168.0.100 is the ip-address of the server

- filename.boot is the name of your boot image.

- Enjoy!